YUVA INTERN

WEEK 1 TECHNICAL REPORT

System Requirements Analysis and
Block Diagram Design

Smart Home Environment Monitoring System

| Field | Details |
| --- | --- |
| Student Name | Heena I |
| Branch | Electrical and Electronics Engineering |
| Task | Week 1 – Embedded System Planning and Architecture |
| Project Type | Embedded / IoT System Design |
| Submission Format | Word Document (.docx) |

Project Focus Design of a practical embedded system that senses indoor temperature, humidity and air quality, processes the readings using an ESP32, displays the results and provides local alerts.

# Table of Contents

1. Introduction

2. Problem Statement

3. Objectives

4. Scope of the System

5. System Requirements Analysis

6. Hardware Requirements

7. Software Requirements

8. Input and Output Specifications

9. Communication Interfaces

10. Power Requirements and Management

11. Real-Time and Performance Requirements

12. System Architecture and Block Diagram

13. Component Description and Design Justification

14. Working Principle

15. Software/Firmware Flow

16. Functional and Non-Functional Requirements

17. Risk, Reliability and Safety Considerations

18. Testing and Validation Plan

19. Planned Work/Effort Breakdown

20. Applications

21. Future Scope

22. Conclusion

23. References

# 1. Introduction

Embedded systems combine hardware and software to perform a dedicated function with predictable operation. Environmental monitoring is a useful embedded-system application because temperature, humidity and air quality can affect comfort, equipment and indoor safety.

This report proposes a Smart Home Environment Monitoring System. The system uses environmental sensors to collect measurements, an ESP32 microcontroller to process the sensor data, an OLED display to present the readings, and LED/buzzer indicators to warn the user when a monitored parameter crosses a predefined threshold. Wi-Fi can be used as an optional communication path for remote monitoring.

The work focuses on system-level planning rather than physical fabrication. The requirements, interfaces, component choices, processing sequence, real-time expectations, power considerations and block-level interactions are defined so that the proposed design can be implemented systematically in a later stage.

# 2. Problem Statement

Homes increasingly contain electronic equipment and enclosed spaces where environmental conditions may change without being noticed. A simple, low-cost and expandable monitoring system is therefore useful for observing indoor temperature, humidity and air-quality trends.

The proposed system addresses this need by continuously acquiring sensor information, processing it locally, presenting understandable readings to the user, and activating an alert when a parameter is outside a selected operating range.

# 3. Objectives

Measure indoor temperature and relative humidity at regular intervals.

Monitor changes in indoor air quality using an MQ-135-type gas sensor.

Process sensor signals using a suitable microcontroller.

Display measured values and system status on a compact OLED display.

Provide immediate local warning through an LED and buzzer when limits are exceeded.

Provide an optional Wi-Fi interface for remote monitoring or future IoT integration.

Develop a modular architecture that can be expanded with additional sensors or actuators.

# 4. Scope of the System

The scope covers the conceptual design of a smart-home environmental monitoring node. It includes sensing, local processing, user indication, optional wireless communication and power-management considerations.

Included: temperature measurement, humidity measurement, basic air-quality monitoring, local display, local alarm, microcontroller processing and optional Wi-Fi.

Not included in the present scope: certified medical/environmental measurements, laboratory-grade gas concentration measurement, automatic HVAC control, or a production-ready cloud backend.

The MQ-135 is treated as a general air-quality indication sensor; exact gas concentration values require appropriate calibration and application-specific validation.

# 5. System Requirements Analysis

## 5.1 Functional Requirements

| ID | Requirement | Description |
| --- | --- | --- |
| FR-01 | Sensor acquisition | Read temperature and humidity from the digital environmental sensor. |
| FR-02 | Air-quality acquisition | Acquire the analog output of the air-quality sensor through an ADC-capable input. |
| FR-03 | Data processing | Validate, process and format sensor readings. |
| FR-04 | Threshold evaluation | Compare readings against configurable warning limits. |
| FR-05 | Display | Show current environmental values and status on the OLED. |
| FR-06 | Alert | Activate LED/buzzer when a warning condition is detected. |
| FR-07 | Wireless communication | Optionally transmit data using Wi-Fi. |
| FR-08 | Fault handling | Indicate sensor or communication faults instead of displaying misleading values. |

## 5.2 System-Level Requirements

| Area | Requirement |
| --- | --- |
| Input | Temperature, humidity and air-quality sensor signals. |
| Processing | ESP32 microcontroller with firmware for acquisition, filtering, threshold checking and control. |
| Output | OLED readings, LED status and buzzer warning. |
| Communication | I²C for OLED and Wi-Fi for optional remote monitoring. |
| Power | Regulated low-voltage DC supply suitable for the selected modules. |
| Operation | Periodic sensing with prompt response to abnormal readings. |
| Expandability | Additional sensors/actuators can be connected through available interfaces. |

# 6. Hardware Requirements

| Component | Role | Reason for Selection |
| --- | --- | --- |
| ESP32 development board | Main controller | Reads sensors, processes data, controls outputs and provides optional Wi-Fi. |
| DHT22 | Temperature & humidity sensor | Provides digital temperature and relative-humidity readings. |
| MQ-135 module | Air-quality indication | Provides an analog signal representing changes in gas/air-quality conditions. |
| 0.96-inch OLED (I²C) | User display | Shows measured values, status and alerts. |
| LED | Visual alert | Indicates normal/warning state. |
| Buzzer | Audible alert | Provides immediate warning when thresholds are crossed. |
| Regulated power supply | Power source | Provides stable DC power to the controller and modules. |
| Connecting wires / breadboard | Interconnection | Used for prototyping and testing. |

## 6.1 Hardware Interface Summary

| Connection | Interface | Purpose |
| --- | --- | --- |
| DHT22 → ESP32 | Digital GPIO | Sensor data |
| MQ-135 → ESP32 | ADC-capable input | Analog air-quality signal |
| ESP32 → OLED | I²C (SDA/SCL) | Display data |
| ESP32 → LED | GPIO | Visual status |
| ESP32 → Buzzer | GPIO through suitable driver if required | Audible alert |
| ESP32 ↔ Wi-Fi | 2.4 GHz wireless | Optional remote monitoring |

# 7. Software Requirements

ESP32-compatible development environment and toolchain.

Embedded C/C++ firmware for sensor acquisition, processing and control.

DHT22 sensor library/driver.

OLED display library/driver for I²C communication.

Wi-Fi library/stack for optional wireless communication.

Serial monitoring tools for debugging and validation.

Version-controlled project files for documentation and future GitHub submission.

## 7.1 Firmware Responsibilities

Initialize GPIO, ADC, I²C, sensor interfaces and display.

Periodically acquire sensor readings.

Check for invalid or missing readings.

Apply simple filtering or averaging where appropriate.

Compare readings with configured thresholds.

Update the OLED display and alert indicators.

Handle optional Wi-Fi connection and data transmission.

Repeat the monitoring cycle continuously.

# 8. Input and Output Specifications

## 8.1 Input Specifications

Temperature: environmental temperature measured by DHT22.

Relative humidity: percentage value measured by DHT22.

Air-quality signal: analog sensor output from the MQ-135 module.

Configuration inputs: threshold values stored in firmware or provided through a future user interface.

## 8.2 Output Specifications

| Output | Expected Information |
| --- | --- |
| OLED | Temperature, humidity, air-quality status and system state. |
| LED | Normal/warning indication. |
| Buzzer | Audible warning during an abnormal condition. |
| Wi-Fi | Optional transmission of sensor data for remote monitoring. |

Design Note Threshold values are application-configurable. The report intentionally avoids treating a generic MQ-135 module as a calibrated instrument for exact gas concentration measurement.

# 9. Communication Interfaces

## 9.1 I²C

The OLED display can communicate with the ESP32 over I²C using the serial data (SDA) and serial clock (SCL) lines. I²C reduces the number of microcontroller pins required for the display and is convenient for compact embedded designs.

## 9.2 GPIO

GPIO connections are used for digital sensor signals and local control of the LED and buzzer. Pin assignments should be finalized during implementation based on the selected ESP32 board and module requirements.

## 9.3 ADC

The MQ-135 module provides an analog output. The ESP32's ADC is used to sample this signal. Proper voltage compatibility and signal conditioning must be checked during physical implementation.

## 9.4 Wi-Fi

The ESP32's integrated Wi-Fi can support future remote monitoring. Sensor data could be sent to a local dashboard or cloud service. The present Week 1 design treats this as an optional communication layer rather than a mandatory dependency for local monitoring.

# 10. Power Requirements and Management

The proposed prototype is intended to operate from a regulated DC supply, such as a USB-based supply appropriate for the selected ESP32 development board. Each module must receive a voltage within its specified operating range.

Use a stable, regulated supply and common ground for connected modules.

Verify the voltage range of every sensor and interface before physical connection.

Avoid powering loads that exceed the microcontroller GPIO drive capability directly.

Use a transistor/driver stage for an actuator if the buzzer or future load requires more current than a GPIO can safely provide.

For battery operation, firmware sleep modes and scheduled sensing can be considered to reduce energy consumption.

Power and signal wiring should be arranged to reduce noise and accidental short circuits.

# 11. Real-Time and Performance Requirements

| Function | Type | Requirement |
| --- | --- | --- |
| Sensing interval | Periodic | Read environmental parameters at regular intervals appropriate to the selected sensors. |
| Display update | Near real-time | Refresh the displayed values after each valid measurement cycle. |
| Alert response | Prompt | Activate the warning output soon after a threshold violation is confirmed. |
| Fault response | Prompt | Show an error/status indication when a sensor read fails. |
| Wi-Fi | Non-blocking where practical | Wireless activity should not unnecessarily stop local sensing and alerting. |

The design should prioritize local sensing and alerting over optional wireless functions. If Wi-Fi is unavailable, the core monitoring and local alert functions should continue to operate.

# 12. System Architecture and Block Diagram

The architecture follows a sensor–processing–output model. Sensors form the input layer, the ESP32 provides the processing and decision-making layer, and the OLED/LED/buzzer form the local output layer. Wi-Fi is an optional communication layer. A regulated power source supplies the system.

DHT22 SENSOR Temperature + Humidity

↓

MQ-135 SENSOR Air-quality indication / analog signal

↓

ESP32 MICROCONTROLLER Sensor acquisition • Processing • Threshold comparison • Control

┌───────────────┬───────────────┬───────────────┐

│               │               │

▼               ▼               ▼

| OLED DISPLAY Values + status | LED + BUZZER Local warning | WI-FI Optional remote monitoring |
| --- | --- | --- |

Power Layer Regulated DC power is distributed to the ESP32 and compatible modules. Power regulation, common grounding and voltage compatibility are verified during implementation.

## 12.1 Data Flow

Sensors observe the indoor environment.

Sensor signals are transferred to the ESP32.

The ESP32 validates and processes the measurements.

Processed data is compared with threshold values.

The OLED is updated with readings and status.

The LED/buzzer are activated when a warning condition is confirmed.

Optional Wi-Fi communication transmits data for remote monitoring.

# 13. Component Description and Design Justification

## 13.1 ESP32 Microcontroller

The ESP32 is selected as the central controller because it provides digital I/O, ADC capability, I²C support and integrated Wi-Fi in a compact development platform. It can therefore support both local monitoring and future IoT expansion.

## 13.2 DHT22 Sensor

DHT22 is selected because a single sensor module provides both temperature and relative-humidity information in digital form. This reduces the number of separate sensing devices required for the basic design.

## 13.3 MQ-135 Sensor

The MQ-135 module is included as a general air-quality indication input. Its analog output allows the microcontroller to observe changes in sensor response. Application-specific calibration is required if quantitative gas concentration is needed.

## 13.4 OLED Display

An I²C OLED provides a compact local user interface while using only a small number of microcontroller pins. It can display multiple parameters and system status in a readable format.

## 13.5 LED and Buzzer

Visual and audible indicators provide an immediate local warning. This is useful even when a user is not continuously looking at the display.

## 13.6 Wi-Fi

Wi-Fi is included as an optional communication layer because the ESP32 supports wireless connectivity. It enables future dashboards, notifications and remote data logging without changing the basic sensing architecture.

# 14. Working Principle

When the system is powered, the ESP32 initializes the sensor, display, communication and output interfaces.

The DHT22 is read to obtain temperature and humidity.

The MQ-135 analog signal is sampled through an appropriate ADC input.

The ESP32 checks whether the sensor readings are valid.

The firmware processes and formats the readings.

The OLED display is refreshed with the latest values and status.

The readings are compared with configured threshold values.

If a warning condition is confirmed, the LED and buzzer are activated.

If Wi-Fi is enabled, the latest data can be transmitted to a remote service.

The system waits for the next sensing interval and repeats the process.

# 15. Software/Firmware Flow

| Flow Stage | Action |
| --- | --- |
| START | Initialize system |
| Initialize Interfaces | GPIO, ADC, I²C, sensors, OLED |
| Read Sensors | Temperature, humidity, air-quality signal |
| Validate Data | Check for missing/invalid readings |
| Process Data | Filtering/formatting |
| Compare Thresholds | Normal or warning condition? |
| Update Outputs | OLED + LED + buzzer |
| Optional Wi-Fi | Transmit data if enabled |
| Repeat | Wait for next sensing cycle |

Firmware Principle The firmware should keep the core sensing and local alert functions independent of the optional Wi-Fi function so that loss of wireless connectivity does not disable basic monitoring.

# 16. Functional and Non-Functional Requirements

## 16.1 Functional Requirements

Acquire environmental measurements.

Process and validate readings.

Display current status.

Detect threshold violations.

Generate local alerts.

Optionally communicate readings over Wi-Fi.

## 16.2 Non-Functional Requirements

| Attribute | Requirement |
| --- | --- |
| Reliability | System should provide consistent readings and identify sensor failures. |
| Usability | Displayed information should be simple and readable. |
| Maintainability | Firmware should use modular functions for sensors, display, alerts and communication. |
| Scalability | Additional sensors or actuators should be addable with minimal architectural changes. |
| Safety | Voltage levels and current requirements must be checked before hardware implementation. |
| Performance | Local sensing and alerting should not be unnecessarily delayed by optional network functions. |

# 17. Risk, Reliability and Safety Considerations

| Risk | Effect | Mitigation |
| --- | --- | --- |
| Incorrect sensor reading | Invalid data may lead to false alerts. | Validate readings; use simple filtering; indicate faults. |
| MQ-135 calibration | Raw sensor response is not automatically a certified gas concentration. | Use it for relative indication unless properly calibrated. |
| Voltage mismatch | Incorrect voltage can damage modules. | Verify module specifications before connection. |
| GPIO overload | Driving a high-current load directly can damage the controller. | Use a suitable driver/transistor where required. |
| Wi-Fi loss | Remote monitoring may stop. | Keep local monitoring and alerts independent. |
| Power instability | Resets or incorrect readings may occur. | Use regulated supply and proper wiring. |

# 18. Testing and Validation Plan

| ID | Test | Validation Method |
| --- | --- | --- |
| T01 | Power-on test | Confirm controller and modules start without abnormal behavior. |
| T02 | Temperature test | Compare DHT22 readings against a suitable reference thermometer. |
| T03 | Humidity test | Check humidity response under different indoor conditions. |
| T04 | Air-quality response | Observe repeatable changes in MQ-135 output under controlled conditions. |
| T05 | Display test | Verify all required values and status messages appear correctly. |
| T06 | Alert test | Force a threshold condition and verify LED/buzzer operation. |
| T07 | Fault test | Disconnect/disable a sensor and verify fault handling. |
| T08 | Wi-Fi test | Verify optional data transmission and behavior during network loss. |

# 19. Planned Work/Effort Breakdown

The task instructions indicate that the work should reflect approximately 30–35 hours of effort. The following is a structured project-work plan totaling 32 hours. It is included to document the intended scope and depth of the work; it should not be treated as a claim of actual time spent if the real effort differs.

| Stage | Work Item | Planned Effort |
| --- | --- | --- |
| 1 | Topic selection and problem definition | 2 hours |
| 2 | Requirement analysis | 5 hours |
| 3 | Hardware/interface selection | 4 hours |
| 4 | Software and firmware architecture planning | 4 hours |
| 5 | Block diagram design and review | 5 hours |
| 6 | Component justification and system integration analysis | 4 hours |
| 7 | Testing/validation planning and risk analysis | 3 hours |
| 8 | Report preparation, formatting and review | 5 hours |

Total planned effort: 32 hours

# 20. Applications

Smart-home environmental monitoring.

Indoor monitoring in classrooms and offices.

Basic environmental observation in laboratories and workshops.

IoT demonstration and embedded-systems education.

Future integration with ventilation or home-automation systems.

# 21. Future Scope

Add a mobile or web dashboard for remote monitoring.

Store historical data for trend analysis.

Add additional environmental sensors such as light, pressure or particulate-matter sensing.

Generate mobile notifications for persistent abnormal conditions.

Add automatic ventilation/fan control through a properly isolated actuator stage.

Introduce battery operation and power-saving modes.

Add secure cloud communication and device authentication for production deployments.

Improve sensor calibration and validation for application-specific accuracy.

# 22. Conclusion

This Week 1 report presents the system-level planning and architecture of a Smart Home Environment Monitoring System. The proposed design combines environmental sensors, an ESP32 microcontroller, a local OLED interface, warning indicators and optional Wi-Fi communication.

The requirements analysis defines the system's inputs, outputs, interfaces, power considerations, real-time expectations, functional requirements and non-functional requirements. The block diagram shows how the major hardware and software functions interact, while the component justification explains why each element is suitable for the proposed application.

The modular architecture provides a clear foundation for later implementation, testing and expansion. The design can be extended with additional sensors, data logging, mobile monitoring and automated control. Overall, the project demonstrates a practical approach to embedded-system planning, requirements analysis and block-level system design.

# 23. References

Espressif Systems, ESP32 Series documentation and technical reference material.

DHT22/AM2302 temperature and humidity sensor datasheet/documentation.

MQ-135 gas/air-quality sensor module datasheet and application documentation.

SSD1306 OLED controller documentation for I²C display modules.

General embedded-systems and Internet-of-Things design references for microcontroller-based monitoring systems.

# Appendix A – Proposed Pin/Interface Mapping

| Signal | Controller Connection | Type |
| --- | --- | --- |
| DHT22 data | ESP32 GPIO | Digital input |
| MQ-135 analog output | ESP32 ADC-capable GPIO | Analog input |
| OLED SDA | ESP32 I²C SDA pin | I²C data |
| OLED SCL | ESP32 I²C SCL pin | I²C clock |
| Status LED | ESP32 GPIO | Digital output |
| Buzzer control | ESP32 GPIO / driver | Digital output |

Final GPIO numbers should be selected during implementation after checking the exact ESP32 development-board model, boot-strapping pins, ADC limitations and the electrical requirements of the selected modules.

# Appendix B – Submission Checklist

Complete Week 1 report prepared in .docx format.

Block diagram included and clearly labelled.

System requirements and component explanations included.

Input/output, communication, power and real-time requirements included.

Testing, risk and future-scope sections included.

Report reviewed for spelling, formatting and readability.

Actual GitHub project URL to be added in the Yuva Intern submission form if required.

Submission description of at least 200 words to be entered in the portal.